Spleen layered dissociation				5/9/18

Protease Mix (make 2 tubes)

7.5 mg/mL Coll. A (75 µL of 100 mg/mL) Sigma, 10103578001

7.5 mg/mL Coll. Type 4 (75 µL of 100 mg/mL) – Worthington, CLS-4

100 µg/mL soybean trypsin inhibitor Sigma, 10109886001

125 U DNAse (5 µL) (Applichem, A3778)

5 mM CaCl2 (5 µL of 1 M CaCl2)

740 µL DPBS	(no Ca, Mg)

Protocol

- Chop tissue coarsely (~30 secs) using razor blade, on ice.

- Add ~12 mg chopped tissue to 1 mL enzyme mix.

- Incubate tube on ice. Triturate 10X every 2 mins and shake every min.

- Monitor digestion using slide/scope.

- After ~10 mins of digestion, let tissue chunks settle for 1 min on ice & remove 80% of supernatant with released cells & filter using 30 µM filter on 50 mL conical, on ice. Rinse filter w/5 mL PBS/0.04% BSA. Save filter and 50 mL conical for next steps.

- Add additional 1 mL enzyme mix to tissue chunks.

- Continue to triturate 10x every 2 minutes and shake every minute while incubating on ice, for 10 additional minutes.

- Remove entire volume and apply to the same 30 µM filter on 50 mL conical. Rinse filter w/5 mL PBS/0.04% BSA.

- Transfer flow-through to 15 mL conical. Spin 650 g for five minutes at 4 C. Remove supernatant (down to ~100 µL).

- Perform RBC lysis: add 1 mL RBC lysis buffer to cells and triturate 10X. Let sit 3 minutes on ice. Add 13 mL PBS/BSA 0.04%

- Spin 650 g for 5 mins at 4 C. Remove supernatant and re-suspend in 1 mL PBS/BSA 0.04%.

- Examine using hemocytometer w/trypan blue and adjust cell concentration to 1000 cells/µL for 10X Chromium or 100 cells/µL for DropSeq.