PAN MRI Processing Pipelines

White Matter Hyperintensities

(PAN-WMH)

# Version Control

| Version | Date (Initials) | Changes |
| --- | --- | --- |
| 1.0 | 10/16/2025 (DK) | Initial Document. |
| 2.0 | 10/17/2025 (DK) | New Release for June 30, 2025 Data Freeze. Additional subjects processed and exceptions updated. |

Contents

Version Control	2

Introduction	4

PAN-WMH-001: LST-AI Analysis	5

LST-AI Processing	5

PAN-WMH-002: Exceptions	7

PAN-WMH-003: References	9

# Introduction

The purpose of this document is to describe the White Matter Hyperintensities (WMH) detection pipelines used for the Magnetic Resonance (MR) Image data collected by the Precision Aging Network (PAN) Project.

# PAN-WMH-001: LST-AI Analysis

## LST-AI Processing

LST-AI has been used to detect the White Matter Lesion segmentation maps. The analysis is performed using public GitHub repository to ensure a reproducible and consistent processing environment. LST-AI is a Python-based deep learning tool developed as a successor to the Lesion Segmentation Toolbox. The following methodological choices have been made.

- Input Requirements: The LST-AI pipeline requires a 3D T1-weighted (T1w) image and a 3D T2 fluid-attenuated inversion recovery (FLAIR) image as inputs. Based on the tool's recommendation, the input images are not skull-stripped prior to processing. (https://github.com/CompImg/LST-AI)

- Automated Preprocessing: LST-AI integrates and automates the necessary preprocessing steps. It internally uses HD-BET for accurate brain extraction and the greedy registration tool for spatial alignment.

- Segmentation and Thresholding: The default analysis mode is used, which performs lesion segmentation and subsequent annotation. A binary lesion mask is generated by applying a threshold of 0.5 to the lesion probability map produced by the tool's deep learning ensemble network. The tool produces WML maps for four distinct brain regions: Periventricular, Juxtacortical, Subcortical, and Infratentorial. For each region, the number of lesions, number of voxels, and total volume in mm³ are computed and reported.’

- Reproducibility Note:
Although LST-AI is designed for standardized processing, the developers have noted some minor inconsistencies in results when running the same example multiple times. This issue is documented in the project’s GitHub repository (Issue #22: “Inconsistent Results When Running the Same Example Multiple Times”). Such variations are expected due to non-deterministic operations within the deep learning framework.

- BIDS-Compatible Execution:
To facilitate scalable and structured analysis, the dataset was organized in BIDS (Brain Imaging Data Structure) format. The Docker version of LST-AI was used in combination with the LST-AI_BIDS wrapper repository (https://github.com/twiltgen/LST-AI_BIDS) , which enables automatic parsing and processing of BIDS-formatted datasets using LST-AI. This setup ensures full reproducibility and compatibility with other BIDS-based tools and pipelines.

A typical LST-AI command line call for the docker version looks as follows:

docker run –rm \

-t \

--gpus "device=2" \

-v subject/session/anat:/lst_input \

-v output_path:/lst_output

-v temp_folder_path:/lst_temp \

jqmcginnis/lst-ai:v1.2.0 \

--t1 t1w_nifti_path \

--flair FLAIR_nifti_path \

--output /lst_output \

--temp /lst_temp \

--device 0 \

--clipping 0.5 99.5 \

--threads 8

# PAN-WMH-002: Exceptions

Certain subjects/sessions could not be successfully processed using the pipeline above or were removed to maintain validity and accuracy. These are delineated below:

| BIDS Participant | BIDS Session | Reason |
| --- | --- | --- |
| sub-HML0021 | ses-01 | No scan acquired |
| sub-HML0028 | ses-01 | No scan acquired |
| sub-HML0053 | ses-01 | No scan acquired |
| sub-HML0074 | ses-01 | FLAIR not acquired |
| sub-HML0114 | ses-01 | No scan acquired |
| sub-HML0145 | ses-01 | No scan acquired |
| sub-HML0148 | ses-01 | FLAIR not acquired |
| sub-HML0168 | ses-01 | No scan acquired |
| sub-HML0180 | ses-01 | No scan acquired |
| sub-HML0183 | ses-01 | No scan acquired |
| sub-HML0185 | ses-01 | No scan acquired |
| sub-HML0186 | ses-01 | No scan acquired |
| sub-HML0272 | ses-01 | No scan acquired |
| sub-HML0278 | ses-01 | No scan acquired |
| sub-HML0283 | ses-01 | No scan acquired |
| sub-HML0288 | ses-01 | No scan acquired |
| sub-HML0292 | ses-01 | No scan acquired |
| sub-HML0293 | ses-01 | No scan acquired |
| sub-HML0294 | ses-01 | No scan acquired |
| sub-HML0295 | ses-01 | No scan acquired |
| sub-HML0331 | ses-01 | No scan acquired |
| sub-HML0333 | ses-01 | No scan acquired |
| sub-HML0344 | ses-01 | No scan acquired |
| sub-HML0355 | ses-01 | No scan acquired |
| sub-HML0358 | ses-01 | No scan acquired |
| sub-HML0363 | ses-01 | No scan acquired |
| sub-HML0367 | ses-01 | No scan acquired |
| sub-HML0369 | ses-01 | No scan acquired |
| sub-HML0374 | ses-01 | No scan acquired |
| sub-HML0381 | ses-01 | No scan acquired |
| sub-HML0396 | ses-01 | No scan acquired |
| sub-HML0405 | ses-01 | No scan acquired |
| sub-HML0407 | ses-01 | T1w not acquired |
| sub-HML0409 | ses-01 | No scan acquired |
| sub-HML0424 | ses-01 | No scan acquired |
| sub-HML0434 | ses-01 | No scan acquired |
| sub-HML0438 | ses-01 | No scan acquired |
| sub-HML0448 | ses-01 | FLAIR not acquired |
| sub-HML0452 | ses-01 | No scan acquired |
| sub-HML0453 | ses-01 | No scan acquired |
| sub-HML0456 | ses-01 | No scan acquired |
| sub-HML0465 | ses-01 | No scan acquired |
| sub-HML0491 | ses-01 | No scan acquired |
| sub-HML0505 | ses-01 | T1w not acquired |
| sub-HML0526 | ses-01 | No scan acquired |
| sub-HML0530 | ses-01 | No scan acquired |
| sub-HML0537 | ses-01 | No scan acquired |
| sub-HML0561 | ses-01 | FLAIR not acquired |
| sub-HML0568 | ses-01 | No scan acquired |
| sub-HML0617 | ses-01 | No scan acquired |
| sub-HML0652 | ses-01 | No scan acquired |
| sub-HML0653 | ses-01 | No scan acquired |
| sub-HML0661 | ses-01 | FLAIR not acquired |
| sub-HML0666 | ses-01 | No scan acquired |
| sub-HML0667 | ses-01 | No scan acquired |
| sub-HML0670 | ses-01 | No scan acquired |
| sub-HML0685 | ses-01 | FLAIR not acquired |
| sub-HML0692 | ses-01 | No scan acquired |
| sub-HML0701 | ses-01 | No scan acquired |
| sub-HML0710 | ses-01 | No scan acquired |
| sub-HML0714 | ses-01 | No scan acquired |
| sub-HML0720 | Ses-01 | FLAIR not acquired |
| sub-HML0731 | ses-01 | FLAIR not acquired |
| sub-HML0738 | ses-01 | No scan acquired |
| sub-HML0772 | ses-01 | No scan acquired |
| sub-HML0775 | ses-01 | No scan acquired |
| sub-HML0788 | ses-01 | No scan acquired |
| sub-HML0792 | ses-01 | No scan acquired |
| sub-HML0862 | ses-01 | No scan acquired |

# PAN-WMH-003: References

Schmidt, P., Gaser, C., Arsic, M., Buck, D., Förschler, A., Berthele, A., ... & Mühlau, M. (2012). An automated tool for detection of FLAIR-hyperintense white-matter lesions in multiple sclerosis. Neuroimage, 59(4), 3774-3783.

Gorgolewski, K. J., Alfaro-Almagro, F., Auer, T., Bellec, P., Capotă, M., Chakravarty, M. M., ... & Poldrack, R. A. (2017). BIDS apps: Improving ease of use, accessibility, and reproducibility of neuroimaging data analysis methods. PLoS computational biology, 13(3), e1005209.