PAN MRI Processing Pipelines

Diffusion Weighted Imaging

(PAN-DWI)

# Version Control

| Version | Date (Initials) | Changes |
| --- | --- | --- |
| 0.1 | 2/18/2025 (CU) | Initial Document. |
| 1.0 | 10/14/2025 (CU) | HML0163 renamed to HML0162 HML0700 included |
| 2.0 | 10/14/2025 | New Release for June 30, 2025 Data Freeze. Additional subjects processed and exceptions updated. |

Contents

Version Control	2

Introduction	5

PAN-DWI-001: Data Acquisition	6

DWI Protocol	6

XNAT	6

Scanner upgrades	6

PAN-DWI-002: BIDS Conversion	7

BIDS Conversion	7

BIDS Sessions	7

BIDS Files	7

DWI	7

Anatomical scans	7

Reverse-Phase Encoded (RPE) Scans	7

Philips specific BIDS additions	8

Phase-encoding directions	8

DWI Slice timing information	8

PAN-DWI-003: HPC Analysis	9

QSIPREP Processing	9

NODDI Processing	11

NODDI White-matter model	11

NODDI Grey-matter model	11

DIFFUSION TENSOR Processing	11

White Matter Tract Segmentation	11

PAN-DWI-004: DWI Derivatives	12

NODDI Gray-matter Derived Summary Measures	12

NODDI White-matter Derived Summary Measures	12

Tensor Gray-matter Derived Summary Measures	13

Tensor White-matter Derived Summary Measures	13

Tractseg WM bundle measures	14

PAN-DWI-005: Exceptions	16

PAN-DWI-006: References	19

PAN-DWI-007: Appendix A : Qsiprep 0.21.4 Bilerplate	20

# Introduction

The purpose of this document is to describe the Diffusion Weighted Imaging (DWI) preprocessing pipelines used for the  Magnetic Resonance (MR) Image data collected by the Precision Aging Network (PAN) Project.

# PAN-DWI-001: Data Acquisition

## DWI Protocol

All four sites collected identical multi-shell acquisitions collected on the sphere. There is just a very minor difference in the number of b=0 volumes between the Siemens and Philips sites.

| Site | Number of b=0 volumes | Number of b=500 volumes | Number of b=1000 volumes | Number of b=2000 volumes | Total Number of volumes |
| --- | --- | --- | --- | --- | --- |
| UA, UM, EU (Siemens) | 13 | 12 | 48 | 64 | 137 |
| JH (Philips) | 12 | 12 | 48 | 64 | 136 |

An additional reverse-phase encoded b0 acquisition was also acquired for susceptibility distortion correction

## XNAT

DICOMS are uploaded to the PAN XNAT using the XNAT Desktop Client or the compressed uploader. Each site has its own unique project on the XNAT. DICOMS at the U of A are directly sent from the scanner to the XNAT.

## Scanner upgrades

On 10/21/2024 UM upgraded from Siemens XA20 to XA50. No changes to the DWI protocol were observed.

On 12/9/2024 UA upgraded from Siemens VE11 to XA30. No changes to the DWI protocol were observed.

# PAN-DWI-002: BIDS Conversion

## BIDS Conversion

BIDS conversion is performed on the XNAT in a custom docker container using Heudiconv 0.10.0 for UA data and using dcm2bids 2.1.6 for the other sites. This initial choice was made because of the variety of DICOM formats (Enhanced DICOMS from UM,EU and JH and Siemens mosaics from U of A).

Anatomical scans (T1 and FLAIR) have also been defaced using pydeface 1.1.0.

## BIDS Sessions

The session tag ses-01 has been used as the default session tag for each subject. On rare occasions subjects returned for a second session if their first scan had issues. Participants HML0102, HML0111, HML0282, HML0502, HML0575 and HML0739 came in for a ses-02 because their first scan had technical difficulties.

## BIDS Files

### DWI

DWI files are located within the BIDS hierarchy in the dwi folder. There will be 4 files for each subject.

| Filename | Description |
| --- | --- |
| sub-HMLDDDD_ses-0X_dwi.nii.gz | DWI scan |
| sub-HMLDDDD_ses-0X_dwi.json | Metadata for DWI |
| sub-HMLDDDD_ses-0X_dwi.bval | Bvals for DWI |
| sub-HMLDDDD_ses-0X_dwi.bvec | Bvecs for DWI |

### Anatomical scans

T1w and FLAIR files are located within the BIDS hierarchy in the anat folder. The defaced files are available for processing and these are named _rec-defaced_T1w and _rec-defaced_FLAIR

### Reverse-Phase Encoded (RPE) Scans

A reverse-phase encoded scan is also acquired and is placed in the fmap folder for use in susceptibility distortion correction of the DWI scans. This is named sub-HMLDDDD_ses-0X_dir-PA_dwi.nii.gz

## Philips specific BIDS additions

Additional processing was required for Philips scans  as the bids conversion process which uses dcm2niix was not able to identify all the necessary metadata from the provided dicoms.

### Phase-encoding directions

The phase encoding direction for the DWI and DWI-RPE scans were inserted into the BIDS metadata for each subject acquired on the Philips scanner.

#### Philips DWI RPE

"PhaseEncodingDirection": "j-",

#### Philips DWI

"PhaseEncodingDirection": "j",

Please note that the DWI and DWI-RPE acquisitions from the Philips scanner have been obtained in the opposite direction to the Siemens sites. This means that the name for the DWI RPE scan sub-HMLDDDD_ses-0X_dir-PA_dwi.nii.gz for the Philips site is misleading as it has actually been collected in the AP direction. This has not been changed at this moment to avoid breaking the preprocessing pipelines. The correct phase-encoding direction is provided in the associated json sidecar.

### DWI Slice timing information

The slice timing information for the DWI and DWI-RPE scans were inserted into the BIDS metadata for each subject acquired on the Philips scanner.

The ExamCard Parameter “Temporal slice spacing” was set to “default” and so the slices are evenly distributed across the TR, taking into account the MultiBand factor. This worked out to be the same slice-timing configuration as the Siemens sites.

# PAN-DWI-003: HPC Analysis

## QSIPREP Processing

QSIPREP version 0.21.4  (strictly speaking QSIPrep 0.21.5.dev0+g36b93fe.d20240504)  is used for processing the diffusion data on the University of Arizona’s High Performance Computer. See the Appendix for the boilerplate. Following below are parameter choices made:

- All the siemens sites use Prescan Normalize and the Philips site uses Constant Level Appearance (CLEAR) which are both methods that correct for non-uniform receiver coil profiles. It was thus decided to run Qsiprep without performing bias field correction with  --b1-biascorrect-stage none

- The DWI acquisition uses Partial Fourier 6/8  acquisition which has been shown to have Gibbs-ringing robustly removed by TORTOISE’s Gibbs. --unringing-method rpg

- Though not explicitly provided in the command line call, the default MP-PCA denoising method dwidenoise was performed.

- FSL’s EDDY and TOPUP were preferred for joint eddy current correction, head motion correction and susceptibility distortion correction. --hmc-model eddy

- The CUDA version of EDDY was used to enable slice-to-volume movement correction and also the correction of susceptibility-by-movement interactions. These are specified in the eddy_params files:

{

"flm": "linear",

"slm": "linear",

"fep": false,

"interp": "spline",

"nvoxhp": 1000,

"fudge_factor": 10,

"dont_sep_offs_move": false,

"dont_peas": false,

"niter": 5,

"method": "jac",

"repol": true,

"num_threads": 1,

"is_shelled": true,

"use_cuda": true,

"cnr_maps": true,

"residuals": true,

"output_type": "NIFTI_GZ",

"estimate_move_by_susceptibility": true,

"mporder": 8,

"args": "--initrand --ol_type=both"

}

- A bids filter file is used to ensure that only session specific anatomical and diffusion data is processed:

{

"dwi": {

"session": "01"

},

"t1w": {

"session": "01"

}

}

The  full command line call looks as follows:

qsiprep BIDS

qsiprep_node

participant

--participant_label HML0551

--separate-all-dwis

--bids-filter-file HML0551_01_bids_filter_file.json

--hmc-model eddy

--unringing-method rpg

--b1-biascorrect-stage none

--eddy-config eddyparams_cuda.json

--mem_mb 50000

--nthreads 10

--fs-license-file license.txt

--write-graph

--output-resolution 2.0

-w qsiprep_work

--anat-modality T1w

--omp-nthreads 1

## NODDI Processing

### NODDI White-matter model

The qsirecon module of QSIPREP version 0.21.4  (strictly speaking QSIPrep 0.21.5.dev0+g36b93fe.d20240504)  was used to fit a white-matter NODDI model (amico-noddi) to the QSIPREP preprocessed outputs. The default NODDI parameters were used.

### NODDI Grey-matter model

Version 2.1.0 of AMICO (https://github.com/daducci/AMICO) was installed into a docker image (aacazxnat/panproc-apps:0.1) and used to fit a grey-matter NODDI model. The parallel diffusivity parameter was set to 0.0011 mm^2/s as described in Guerrero et al (2019) to more accurately model the tissue compartments in gray matter.

## DIFFUSION TENSOR Processing

Fractional Anisotropy (FA),Mean Diffusivity (MD),  Radial Diffusivity (RD) and Axial Diffusivity (AD) were all calculated from QSIPREP preprocessed outputs using Mrtrix version 3.0.4

## White Matter Tract Segmentation

White matter bundle segmentation for each subject was performed on the preprocessed QSIPREP outputs using Tractseg (https://github.com/MIC-DKFZ/TractSeg) version 2.8 which is conveniently available in a docker image (wasserth/tractseg_container:master)

# PAN-DWI-004: DWI Derivatives

Derivative measures based on the preprocessing steps mentioned above are shared in the coree_chen_dwi_measures table.

## NODDI Gray-matter Derived Summary Measures

Using the NODDI measures derived by the Amico-noddi gray matter model  additional masking has been preformed using  the Gray matter calculated using FSL’s FAST algorithm (as part of the fsl_anat anatomical processing pipeline  https://web.mit.edu/fsl_v5.0.10/fsl/doc/wiki/fsl_anat.html) and the cortical masks derived by Freesurfer’s surface pipeline using mris_volmask (https://surfer.nmr.mgh.harvard.edu/fswiki/mris%25volmask )  to obtain summaries of Free Water Fraction (FWF), Neurite Density Index (NDI) and the Orientation Dispersion Index (ODI)  in cortical GM These measures are calculated in the qsiprep native space.

| Measure | Description |
| --- | --- |
| noddi_gm.gmcort.fit_FWF.Cerebral_Cortex | NODDI GM Free Water Fraction (FWF) in Cortical GM |
| noddi_gm.gmcort.fit_NDI.Cerebral_Cortex | NODDI GM Neurite Density Index (NDI) in Cortical GM |
| noddi_gm.gmcort.fit_ODI.Cerebral_Cortex | NODDI GM Orientation Dispersion Index (ODI) in Cortical GM |
| noddi_gm.gmhemi.fit_FWF.Left_Cerebral_Cortex | NODDI GM FWF left Cortical GM |
| noddi_gm.gmhemi.fit_FWF.Right_Cerebral_Cortex | NODDI GM FWF Right Cortical GM |
| noddi_gm.gmhemi.fit_NDI.Left_Cerebral_Cortex | NODDI GM NDI left Cortical GM |
| noddi_gm.gmhemi.fit_NDI.Right_Cerebral_Cortex | NODDI GM NDI Right Cortical GM |
| noddi_gm.gmhemi.fit_ODI.Left_Cerebral_Cortex | NODDI GM ODI left Cortical GM |
| noddi_gm.gmhemi.fit_ODI.Right_Cerebral_Cortex | NODDI GM ODI Right Cortical GM |

## NODDI White-matter Derived Summary Measures

Using the NODDI measures derived by QSIPREP’s  Qsirecon white matter model,  additional masking has been performed as described above “NODDI Gray-matter Derived Summary Measures” using White matter masks instead. Note that QSIPREP refers to the FWF as ISOVF, the NDI as ICVF and the ODI as OD.

| Measure | Description |
| --- | --- |
| noddi_wm.wmhemi.desc_ICVF.Left_Cerebral_White_Matter | NODDI NDI left Cerebral WM |
| noddi_wm.wmhemi.desc_ICVF.Right_Cerebral_White_Matter | NODDI NDI Right Cerebral WM |
| noddi_wm.wmhemi.desc_ISOVF.Left_Cerebral_White_Matter | NODDI FWF left Cerebral WM |
| noddi_wm.wmhemi.desc_ISOVF.Right_Cerebral_White_Matter | NODDI FWF Right Cerebral WM |
| noddi_wm.wmhemi.desc_OD.Left_Cerebral_White_Matter | NODDI ODI left Cerebral WM |
| noddi_wm.wmhemi.desc_OD.Right_Cerebral_White_Matter | NODDI ODI Right Cerebral WM |
| noddi_wm.wmintra.desc_ICVF.Cerebral_White_Matter | NODDI NDI in Cerebral WM |
| noddi_wm.wmintra.desc_ISOVF.Cerebral_White_Matter | NODDI FWF in Cerebral WM |
| noddi_wm.wmintra.desc_OD.Cerebral_White_Matter | NODDI ODI in Cerebral WM |

## Tensor Gray-matter Derived Summary Measures

Using the Tensor measures derived by Mrtrix  additional masking has been performed using  the Gray matter mask calculated using FSL’s FAST algorithm (as part of the fsl_anat anatomical processing pipeline  https://web.mit.edu/fsl_v5.0.10/fsl/doc/wiki/fsl_anat.html) and the cortical masks derived by Freesurfer’s surface pipeline using mris_volmask (https://surfer.nmr.mgh.harvard.edu/fswiki/mris%25volmask )  to obtain summaries of Axial Diffusivity (AD), mean Diffusivity/ mean Apparent Diffusion Coefficient (ADC), Fractional Anisotropy (FA) and the Radial Diffusivity (RD) in cortical GM These measures are calculated in the qsiprep native space.

| Measure | Description |
| --- | --- |
| tensor_gm.gmcort.desc_ad.Cerebral_Cortex | Axial Diffusivity (AD) in Cortical GM |
| tensor_gm.gmcort.desc_adc.Cerebral_Cortex | Mean Apparent Diffusion Coefficient (ADC) in Cortical GM |
| tensor_gm.gmcort.desc_fa.Cerebral_Cortex | Fractional Anisotropy (AD) in Cortical GM |
| tensor_gm.gmcort.desc_rd.Cerebral_Cortex | Radial Diffusivity (RD) in Cortical GM |
| tensor_gm.gmhemi.desc_ad.Left_Cerebral_Cortex | AD in Left Cortical GM |
| tensor_gm.gmhemi.desc_ad.Right_Cerebral_Cortex | AD in Right Cortical GM |
| tensor_gm.gmhemi.desc_adc.Left_Cerebral_Cortex | ADC in Left Cortical GM |
| tensor_gm.gmhemi.desc_adc.Right_Cerebral_Cortex | ADC in Right Cortical GM |
| tensor_gm.gmhemi.desc_fa.Left_Cerebral_Cortex | FA in Left Cortical GM |
| tensor_gm.gmhemi.desc_fa.Right_Cerebral_Cortex | FA in Right Cortical GM |
| tensor_gm.gmhemi.desc_rd.Left_Cerebral_Cortex | RD in Left Cortical GM |
| tensor_gm.gmhemi.desc_rd.Right_Cerebral_Cortex | RD in Right Cortical GM |

## Tensor White-matter Derived Summary Measures

Using the Tensor measures derived by Mrtrix  additional masking has been performed as described above “Tensor Gray-matter Derived Summary Measures” using white matter masks instead.

| Measure | Description |
| --- | --- |
| tensor_wm.wmhemi.desc_ad.Left_Cerebral_White_Matter | AD in Left Cerebral WM |
| tensor_wm.wmhemi.desc_ad.Right_Cerebral_White_Matter | AD in Right Cerebral WM |
| tensor_wm.wmhemi.desc_adc.Left_Cerebral_White_Matter | ADC in Left Cerebral WM |
| tensor_wm.wmhemi.desc_adc.Right_Cerebral_White_Matter | ADC in Right Cerebral WM |
| tensor_wm.wmhemi.desc_fa.Left_Cerebral_White_Matter | FA in Left Cerebral WM |
| tensor_wm.wmhemi.desc_fa.Right_Cerebral_White_Matter | FA in Right Cerebral WM |
| tensor_wm.wmhemi.desc_rd.Left_Cerebral_White_Matter | RD in Left Cerebral WM |
| tensor_wm.wmhemi.desc_rd.Right_Cerebral_White_Matter | RD in Right Cerebral WM |
| tensor_wm.wmintra.desc_ad.Cerebral_White_Matter | AD in Cerebral WM |
| tensor_wm.wmintra.desc_adc.Cerebral_White_Matter | ADC in Cerebral WM |
| tensor_wm.wmintra.desc_fa.Cerebral_White_Matter | FA in Cerebral WM |
| tensor_wm.wmintra.desc_rd.Cerebral_White_Matter | RD in Cerebral WM |

## Tractseg WM bundle measures

In addition to the above 38 WM tracts have been determined for each subject and mean measures of AD, ADC, FA, RD, NDI, ODI and FWF have been calculated for each tract for each subject.  For the Neurite Denisty Index (NDI) this is displayed below.

| Measure | Description |
| --- | --- |
| noddi_wm.tractseg_native.desc_ICVF.AF_left | NODDI NDI Arcuate fasciculus (AF) Left |
| noddi_wm.tractseg_native.desc_ICVF.AF_right | NODDI NDI Arcuate fasciculus (AF) Right |
| noddi_wm.tractseg_native.desc_ICVF.ATR_left | NODDI NDI Anterior Thalamic Radiation (ATR) Left |
| noddi_wm.tractseg_native.desc_ICVF.ATR_right | NODDI NDI Anterior Thalamic Radiation (ATR) Right |
| noddi_wm.tractseg_native.desc_ICVF.CC_ant_midbody | NODDI NDI Corpus Collusum (CC) Ant. Midbody |
| noddi_wm.tractseg_native.desc_ICVF.CC_genu | NODDI NDI Corpus Collusum (CC) Genu |
| noddi_wm.tractseg_native.desc_ICVF.CC_isthmus | NODDI NDI Corpus Collusum (CC) Isthmus |
| noddi_wm.tractseg_native.desc_ICVF.CC_post_midbody | NODDI NDI Corpus Collusum (CC) Post. Midbody |
| noddi_wm.tractseg_native.desc_ICVF.CC_rostral_body | NODDI NDI Corpus Collusum (CC) Rostral body |
| noddi_wm.tractseg_native.desc_ICVF.CC_rostrum | NODDI NDI Corpus Collusum (CC) Rostrum |
| noddi_wm.tractseg_native.desc_ICVF.CC_splenium | NODDI NDI Corpus Collusum (CC) Splenium |
| noddi_wm.tractseg_native.desc_ICVF.CG_left | NODDI NDI Cingulum (CG) Left |
| noddi_wm.tractseg_native.desc_ICVF.CG_right | NODDI NDI Cingulum (CG) Right |
| noddi_wm.tractseg_native.desc_ICVF.CST_left | NODDI NDI Corticospinal Tract (CST) Left |
| noddi_wm.tractseg_native.desc_ICVF.CST_right | NODDI NDI Corticospinal Tract (CST) Right |
| noddi_wm.tractseg_native.desc_ICVF.FX_left | NODDI NDI Fornix (FX) Left |
| noddi_wm.tractseg_native.desc_ICVF.FX_right | NODDI NDI Fornix (FX) Right |
| noddi_wm.tractseg_native.desc_ICVF.ICP_left | NODDI NDI Inferior cerebellar peduncle (ICP) Left |
| noddi_wm.tractseg_native.desc_ICVF.ICP_right | NODDI NDI Inferior cerebellar peduncle (ICP) Right |
| noddi_wm.tractseg_native.desc_ICVF.ILF_left | NODDI NDI Inferior longitudinal fasciculus (ILF) Left |
| noddi_wm.tractseg_native.desc_ICVF.ILF_right | NODDI NDI Inferior longitudinal fasciculus (ILF) Right |
| noddi_wm.tractseg_native.desc_ICVF.MCP | NODDI NDI Middle cerebellar peduncle (MCP) |
| noddi_wm.tractseg_native.desc_ICVF.MLF_left | NODDI NDI Middle longitudinal fasciculus (MLF) Left |
| noddi_wm.tractseg_native.desc_ICVF.MLF_right | NODDI NDI Middle longitudinal fasciculus (MLF) Right |
| noddi_wm.tractseg_native.desc_ICVF.OR_left | NODDI NDI Optic Radiation (OR) Left |
| noddi_wm.tractseg_native.desc_ICVF.OR_right | NODDI NDI Optic Radiation (OR) Right |
| noddi_wm.tractseg_native.desc_ICVF.SCP_left | NODDI NDI Superior cerebellar peduncle (SCP) Left |
| noddi_wm.tractseg_native.desc_ICVF.SCP_right | NODDI NDI Superior cerebellar peduncle (SCP) Right |
| noddi_wm.tractseg_native.desc_ICVF.SLF_III_left | NODDI NDI Superior longitudinal fasciculus III (SLF_III) Left |
| noddi_wm.tractseg_native.desc_ICVF.SLF_III_right | NODDI NDI Superior longitudinal fasciculus III (SLF_III) Right |
| noddi_wm.tractseg_native.desc_ICVF.SLF_II_left | NODDI NDI Superior longitudinal fasciculus II (SLF_II) Left |
| noddi_wm.tractseg_native.desc_ICVF.SLF_II_right | NODDI NDI Superior longitudinal fasciculus II (SLF_II) Right |
| noddi_wm.tractseg_native.desc_ICVF.SLF_I_left | NODDI NDI Superior longitudinal fasciculus I (SLF_I) Left |
| noddi_wm.tractseg_native.desc_ICVF.SLF_I_right | NODDI NDI Superior longitudinal fasciculus I (SLF_I) Right |
| noddi_wm.tractseg_native.desc_ICVF.STR_left | NODDI NDI Superior Thalamic Radiation (STR) Left |
| noddi_wm.tractseg_native.desc_ICVF.STR_right | NODDI NDI Superior Thalamic Radiation (STR) Right |
| noddi_wm.tractseg_native.desc_ICVF.UF_left | NODDI NDI Uncinate fasciculus (UF) Left |
| noddi_wm.tractseg_native.desc_ICVF.UF_right | NODDI NDI Uncinate fasciculus (UF) Right |

# PAN-DWI-005: Exceptions

Certain subjects/sessions could not be successfully processed using the pipeline above or were removed to maintain validity and accuracy. These are delineated below:

| BIDS Participant | BIDS Session | Reason |
| --- | --- | --- |
| sub-HML0015 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0021 | ses-01 | No scan acquired |
| sub-HML0028 | ses-01 | No scan acquired |
| sub-HML0037 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0053 | ses-01 | No scan acquired |
| sub-HML0060 | ses-01 | DWI not acquired. |
| sub-HML0068 | ses-01 | DWI RPE not acquired correctly. Subject did come in for a session 002 but a T1w image was not collected |
| sub-HML0070 | ses-01 | DWI RPE not acquired correctly. Subject did come in for a session 002 but a T1w image was not collected. |
| sub-HML0097 | ses-01 | DWI RPE not acquired correctly. |
| sub-HML0104 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0114 | ses-01 | No scan acquired |
| sub-HML0126 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0145 | ses-01 | No scan acquired |
| sub-HML0152 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0166 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0168 | ses-01 | No scan acquired |
| sub-HML0180 | ses-01 | No scan acquired |
| sub-HML0183 | ses-01 | No scan acquired |
| sub-HML0185 | ses-01 | No scan acquired |
| sub-HML0186 | ses-01 | No scan acquired |
| sub-HML0194 | ses-01 | DWI and DWI RPE not acquired. |
| sub-HML0264 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0272 | ses-01 | No scan acquired |
| sub-HML0278 | ses-01 | No scan acquired |
| sub-HML0283 | ses-01 | No scan acquired |
| sub-HML0288 | ses-01 | No scan acquired |
| sub-HML0292 | ses-01 | No scan acquired |
| sub-HML0293 | ses-01 | No scan acquired |
| sub-HML0294 | ses-01 | No scan acquired |
| sub-HML0295 | ses-01 | No scan acquired |
| sub-HML0331 | ses-01 | No scan acquired |
| sub-HML0333 | ses-01 | No scan acquired |
| sub-HML0344 | ses-01 | No scan acquired |
| sub-HML0350 | ses-01 | DWI RPE not acquired correctly. |
| sub-HML0355 | ses-01 | No scan acquired |
| sub-HML0358 | ses-01 | No scan acquired |
| sub-HML0359 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0363 | ses-01 | No scan acquired |
| sub-HML0367 | ses-01 | No scan acquired |
| sub-HML0369 | ses-01 | No scan acquired |
| sub-HML0372 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0374 | ses-01 | No scan acquired |
| sub-HML0381 | ses-01 | No scan acquired |
| sub-HML0396 | ses-01 | No scan acquired |
| sub-HML0405 | ses-01 | No scan acquired |
| sub-HML0406 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0407 | ses-01 | DWI not acquired. |
| sub-HML0409 | ses-01 | No scan acquired |
| sub-HML0416 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0424 | ses-01 | No scan acquired |
| sub-HML0430 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0434 | ses-01 | No scan acquired |
| sub-HML0438 | ses-01 | No scan acquired |
| sub-HML0452 | ses-01 | No scan acquired |
| sub-HML0453 | ses-01 | No scan acquired |
| sub-HML0454 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0456 | ses-01 | No scan acquired |
| sub-HML0463 | ses-01 | DWI not acquired. |
| sub-HML0465 | ses-01 | No scan acquired |
| sub-HML0466 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0491 | ses-01 | No scan acquired |
| sub-HML0505 | ses-01 | DWI not acquired. |
| sub-HML0519 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0526 | ses-01 | No scan acquired |
| sub-HML0530 | ses-01 | No scan acquired |
| sub-HML0537 | ses-01 | No scan acquired |
| sub-HML0568 | ses-01 | No scan acquired |
| sub-HML0579 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0616 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0617 | ses-01 | No scan acquired |
| sub-HML0652 | ses-01 | No scan acquired |
| sub-HML0653 | ses-01 | No scan acquired |
| sub-HML0654 | Ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0661 | ses-01 | DWI only partially acquired. |
| sub-HML0666 | ses-01 | No scan acquired |
| sub-HML0667 | ses-01 | No scan acquired |
| sub-HML0670 | ses-01 | No scan acquired |
| sub-HML0692 | ses-01 | No scan acquired |
| sub-HML0701 | ses-01 | No scan acquired |
| sub-HML0710 | ses-01 | No scan acquired |
| sub-HML0714 | ses-01 | No scan acquired |
| sub-HML0738 | ses-01 | No scan acquired |
| sub-HML0772 | ses-01 | No scan acquired |
| sub-HML0775 | ses-01 | No scan acquired |
| sub-HML0788 | ses-01 | No scan acquired |
| sub-HML0792 | ses-01 | No scan acquired |
| sub-HML0810 | ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0815 | ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0859 | ses-01 | Tractseg failure during WM tract segmentation |
| sub-HML0862 | ses-01 | No scan acquired |

# PAN-DWI-006: References

Guerrero, J. M., Adluru, N., Bendlin, B. B., Goldsmith, H. H., Schaefer, S. M., Davidson, R. J., ... & Alexander, A. L. (2019). Optimizing the intrinsic parallel diffusivity in NODDI: An extensive empirical evaluation. PloS one, 14(9), e0217118.

Cieslak, M., Cook, P. A., He, X., Yeh, F. C., Dhollander, T., Adebimpe, A., ... & Satterthwaite, T. D. (2021). QSIPrep: an integrative platform for preprocessing and reconstructing diffusion MRI data. Nature methods, 18(7), 775-778.

Daducci, A., Canales-Rodríguez, E. J., Zhang, H., Dyrby, T. B., Alexander, D. C., & Thiran, J. P. (2015). Accelerated microstructure imaging via convex optimization (AMICO) from diffusion MRI data. Neuroimage, 105, 32-44.

Tournier, J. D., Calamante, F., & Connelly, A. (2012). MRtrix: diffusion tractography in crossing fiber regions. International journal of imaging systems and technology, 22(1), 53-66.

Zhang, H., Schneider, T., Wheeler-Kingshott, C. A., & Alexander, D. C. (2012). NODDI: practical in vivo neurite orientation dispersion and density imaging of the human brain. Neuroimage, 61(4), 1000-1016.

Wasserthal, J., Neher, P., & Maier-Hein, K. H. (2018). TractSeg-Fast and accurate white matter tract segmentation. NeuroImage, 183, 239-253.

Jenkinson, M., Beckmann, C. F., Behrens, T. E. J., Woolrich, M. W., & Smith, S. M. (2012). FSL. NeuroImage, 62(2), 782–790. https://doi.org/10.1016/j.neuroimage.2011.09.015

Fischl, B. (2012). FreeSurfer. Neuroimage, 62(2), 774-781.

# PAN-DWI-007: Appendix A : Qsiprep 0.21.4 Boilerplate

Preprocessing was performed using QSIPrep 0.21.5.dev0+g36b93fe.d20240504, which is based on Nipype 1.8.6 (Gorgolewski et al. (2011); Gorgolewski et al. (2018); RRID:SCR_002502).

#### Anatomical data preprocessing

The T1-weighted (T1w) image was corrected for intensity non-uniformity (INU) using N4BiasFieldCorrection (Tustison et al. 2010, ANTs 2.4.3), and used as an anatomical reference throughout the workflow. The anatomical reference image was reoriented into AC-PC alignment via a 6-DOF transform extracted from a full Affine registration to the MNI152NLin2009cAsym template. A full nonlinear registration to the template from AC-PC space was estimated via symmetric nonlinear registration (SyN) using antsRegistration. Brain extraction was performed on the T1w image using SynthStrip (Hoopes et al. 2022) and automated segmentation was performed using SynthSeg (Billot, Greve, et al. 2023, @synthseg2) from FreeSurfer version 7.3.1.

#### Diffusion data preprocessing

Any images with a b-value less than 100 s/mm^2 were treated as a b=0 image. MP-PCA denoising as implemented in MRtrix3’s dwidenoise(Veraart et al. 2016) was applied with a 5-voxel window. When phase data were available, this was done on complex-valued data. After MP-PCA, Gibbs unringing was performed using TORTOISE’s Gibbs (Lee, Novikov, and Fieremans 2021). Following unringing, the mean intensity of the DWI series was adjusted so all the mean intensity of the b=0 images matched across eachseparate DWI scanning sequence.

FSL (version None)’s eddy was used for head motion correction and Eddy current correction (Andersson and Sotiropoulos 2016). Eddy was configured with a q-space smoothing factor of 10, a total of 5 iterations, and 1000 voxels used to estimate hyperparameters. A linear first level model and a linear second level model were used to characterize Eddy current-related spatial distortion. q-space coordinates were forcefully assigned to shells. Field offset was attempted to be separated from subject movement. Shells were aligned post-eddy. Eddy’s outlier replacement was run (Andersson et al. 2016). Data were grouped by slice, only including values from slices determined to contain at least 250 intracerebral voxels. Groups deviating by more than 4 standard deviations from the prediction had their data replaced with imputed values. Slice-to-volume correction was estimated with temporal order 8, 5 iterations, trilinear interpolation and lambda=1.000 (Andersson et al. 2017).

Data was collected with reversed phase-encode blips, resulting in pairs of images with distortions going in opposite directions. FSL’s TOPUP (Andersson, Skare, and Ashburner 2003) was used to estimate a susceptibility-induced off-resonance field based on b=0 reference images with reversed phase encoding directions. The TOPUP-estimated fieldmap was incorporated into the Eddy current and head motion correction interpolation. Dynamic susceptibility distortion correction was applied with 10 iterations, lambda=10.00 and spline knot-spacing of 10.00mm (Andersson et al. 2018). Final interpolation was performed using the jac method.

Several confounding time-series were calculated based on the preprocessed DWI: framewise displacement (FD) using the implementation in Nipype (following the definitions by Power et al. 2014). The head-motion estimates calculated in the correction step were also placed within the corresponding confounds file. Slicewise cross correlation was also calculated. The DWI time-series were resampled to ACPC, generating a preprocessed DWI run in ACPC space with 2mm isotropic voxels.

Many internal operations of QSIPrep use Nilearn 0.10.1 (Abraham et al. 2014, RRID:SCR_001362) and Dipy (Garyfallidis et al. 2014). For more details of the pipeline, see the section corresponding to workflows in QSIPrep’s documentation.

#### References

Abraham, Alexandre, Fabian Pedregosa, Michael Eickenberg, Philippe Gervais, Andreas Mueller, Jean Kossaifi, Alexandre Gramfort, Bertrand Thirion, and Gael Varoquaux. 2014. “Machine Learning for Neuroimaging with Scikit-Learn.” Frontiers in Neuroinformatics 8. https://doi.org/10.3389/fninf.2014.00014.

Andersson, Jesper LR, Mark S Graham, Ivana Drobnjak, Hui Zhang, and Jon Campbell. 2018. “Susceptibility-Induced Distortion That Varies Due to Motion: Correction in Diffusion Mr Without Acquiring Additional Data.” Neuroimage 171. Elsevier: 277–95.

Andersson, Jesper LR, Mark S Graham, Ivana Drobnjak, Hui Zhang, Nicola Filippini, and Matteo Bastiani. 2017. “Towards a Comprehensive Framework for Movement and Distortion Correction of Diffusion Mr Images: Within Volume Movement.” Neuroimage 152. Elsevier: 450–66.

Andersson, Jesper LR, Mark S Graham, Enikő Zsoldos, and Stamatios N Sotiropoulos. 2016. “Incorporating Outlier Detection and Replacement into a Non-Parametric Framework for Movement and Distortion Correction of Diffusion Mr Images.” Neuroimage 141. Elsevier: 556–72.

Andersson, Jesper LR, Stefan Skare, and John Ashburner. 2003. “How to Correct Susceptibility Distortions in Spin-Echo Echo-Planar Images: Application to Diffusion Tensor Imaging.” Neuroimage 20 (2). Elsevier: 870–88.

Andersson, Jesper LR, and Stamatios N Sotiropoulos. 2016. “An Integrated Approach to Correction for Off-Resonance Effects and Subject Movement in Diffusion Mr Imaging.” Neuroimage 125. Elsevier: 1063–78.

Billot, Benjamin, Douglas N Greve, Oula Puonti, Axel Thielscher, Koen Van Leemput, Bruce Fischl, Adrian V Dalca, Juan Eugenio Iglesias, and others. 2023. “SynthSeg: Segmentation of Brain Mri Scans of Any Contrast and Resolution Without Retraining.” Medical Image Analysis 86. Elsevier: 102789.

Billot, Benjamin, Colin Magdamo, You Cheng, Steven E Arnold, Sudeshna Das, and Juan Eugenio Iglesias. 2023. “Robust Machine Learning Segmentation for Large-Scale Analysis of Heterogeneous Clinical Brain Mri Datasets.” Proceedings of the National Academy of Sciences 120 (9). National Acad Sciences: e2216399120.

Garyfallidis, Eleftherios, Matthew Brett, Bagrat Amirbekian, Ariel Rokem, Stefan Van Der Walt, Maxime Descoteaux, and Ian Nimmo-Smith. 2014. “Dipy, a Library for the Analysis of Diffusion Mri Data.” Frontiers in Neuroinformatics 8. Frontiers: 8.

Gorgolewski, K., C. D. Burns, C. Madison, D. Clark, Y. O. Halchenko, M. L. Waskom, and S. Ghosh. 2011. “Nipype: A Flexible, Lightweight and Extensible Neuroimaging Data Processing Framework in Python.” Frontiers in Neuroinformatics 5: 13. https://doi.org/10.3389/fninf.2011.00013.

Gorgolewski, Krzysztof J., Oscar Esteban, Christopher J. Markiewicz, Erik Ziegler, David Gage Ellis, Michael Philipp Notter, Dorota Jarecka, et al. 2018. “Nipype.” Software. Zenodo. https://doi.org/10.5281/zenodo.596855.

Hoopes, Andrew, Jocelyn S Mora, Adrian V Dalca, Bruce Fischl, and Malte Hoffmann. 2022. “SynthStrip: Skull-Stripping for Any Brain Image.” NeuroImage 260. Elsevier: 119474.

Lee, Hong-Hsi, Dmitry S Novikov, and Els Fieremans. 2021. “Removal of Partial Fourier-Induced Gibbs (Rpg) Ringing Artifacts in Mri.” Magnetic Resonance in Medicine 86 (5). Wiley Online Library: 2733–50.

Power, Jonathan D., Anish Mitra, Timothy O. Laumann, Abraham Z. Snyder, Bradley L. Schlaggar, and Steven E. Petersen. 2014. “Methods to Detect, Characterize, and Remove Motion Artifact in Resting State fMRI.” NeuroImage 84 (Supplement C): 320–41. https://doi.org/10.1016/j.neuroimage.2013.08.048.

Tustison, N. J., B. B. Avants, P. A. Cook, Y. Zheng, A. Egan, P. A. Yushkevich, and J. C. Gee. 2010. “N4ITK: Improved N3 Bias Correction.” IEEE Transactions on Medical Imaging 29 (6): 1310–20. https://doi.org/10.1109/TMI.2010.2046908.

Veraart, Jelle, Dmitry S Novikov, Daan Christiaens, Benjamin Ades-Aron, Jan Sijbers, and Els Fieremans. 2016. “Denoising of Diffusion Mri Using Random Matrix Theory.” NeuroImage 142. Elsevier: 394–406.