PAN MRI Processing Pipelines

Arterial Spin Labelling

(PAN-ASL)

# Version Control

| Version | Date (Initials) | Changes |
| --- | --- | --- |
| 0.1 | 2/5/2025 (CU) | Initial Document. |
| 1.0 | 10/14/2025 (CU) | HML0163 renamed to HML0162 HML0700 included |
| 2.0 | 10/14/2025 | New Release for June 30, 2025 Data Freeze. Additional subjects processed and exceptions updated. |

Contents

Version Control	2

Introduction	4

PAN-ASL-001: Data Acquisition	5

PASL and PCASL Protocols	5

ASL plus M0	5

XNAT	5

Scanner upgrades	5

Chemical shift artefact	5

PAN-ASL-002: BIDS Conversion	6

BIDS Conversion	6

BIDS Sessions	6

BIDS Files	6

ASL	6

Anatomical scans	6

Field Maps	7

Additional ASL fields	7

PASL	7

PCASL	7

Effective Echo Spacing	8

PAN-ASL-003: HPC Analysis	9

BASIL Processing	9

PASL	10

PCASL	10

PAN-ASL-004: ASL Derivatives	12

BASIL CBF Derived Summary Measures	12

BASIL CBF Measures in Arterial Territories	12

PAN-ASL-005: Exceptions	14

PAN-ASL-006: References	17

# Introduction

The purpose of this document is to describe the Arterial Spin Labelling (ASL) preprocessing pipelines used for the  Magnetic Resonance (MR) Image data collected by the Precision Aging Network (PAN) Project.

# PAN-ASL-001: Data Acquisition

## PASL and PCASL Protocols

The three Siemens sites (UA, UM and EU) acquire ASL data using Pulsed Arterial Spin Labelling (PASL). JH uses a Philips scanner and has acquired ASL using Pseudo-continuous  Arterial Spin Labelling (PCASL). A separate M0 (proton-density) image is acquired in all sites for calibration.

## ASL plus M0

At the beginning of the project an initial acquisition was used that collected ASL and included an M0. This was collected for 6 subjects (HML0046, HML0048, HML0049, HML0050, HML0051 and HML0195) and then discontinued. This data is only available at the UA site and is not included in this data release.

## XNAT

DICOMS are uploaded to the PAN XNAT using the XNAT Desktop Client or the compressed uploader. Each site has its own unique project on the XNAT. DICOMS at the U of A are directly sent from the scanner to the XNAT.

## Scanner upgrades

On 10/21/2024 UM upgraded from Siemens XA20 to XA50. The main impact of this was an extra volume acquired for both the M0 (from 4 to 5 volumes) and the Control-Label scans (from 20 to 21 volumes) which added an extra minute to the scan. In both cases the first volume appears to be a dummy scan and has been removed after BIDS conversion.

On 12/9/2024 UA upgraded from Siemens VE11 to XA30. No changes to the timing and volumes acquired were observed.

## Chemical shift artefact

A chemical shift artefact is prominent in the PASL acquisition. Using scalp surfaces generated by the MNE package’s make_scalp_surfaces,  masks have been created which are used to exclude affected voxels from calculated measures. When these masks are used in a reported measure then the field name will contain a suffix with “chemshift” in it.

# PAN-ASL-002: BIDS Conversion

## BIDS Conversion

BIDS conversion is performed on the XNAT in a custom docker container using Heudiconv 0.10.0 for UA data and using dcm2bids 2.1.6 for the other sites. This initial choice was made because of the variety of DICOM formats (Enhanced DICOMS from UM,EU and JH and Siemens mosaics from U of A).

Anatomical scans (T1 and FLAIR) have also been defaced using pydeface 1.1.0.

The first 2 echoes of the QSM acquisition MEGRE have been made available for field map creation.

## BIDS Sessions

The session tag ses-01 has been used as the default session tag for each subject. On rare occasions subjects returned for a second session if their first scan had issues. Participants HML0301 and HML0509 came in for a ses-02 because their first scan had technical difficulties.

## BIDS Files

### ASL

ASL files are located within the BIDS hierarchy in the perf folder. There will be 5 files for each subject. For PASL sites i.e. UM, UA and EU the tag acq-prod will be used.

For PCASL site (JH) the tag used will be acq-pcasl.

| Filename | Description |
| --- | --- |
| sub-HMLDDDD_ses-0X_<tag>_asl.nii.gz | ASL scan |
| sub-HMLDDDD_ses-0X_<tag>_asl.json | Metadata for ASL |
| sub-HMLDDDD_ses-0X_<tag>_asl.context | Order of acquisition of Control and Label volumes |
| sub-HMLDDDD_ses-0X_<tag>_m0scan.nii.gz | M0 scan |
| sub-HMLDDDD_ses-0X_<tag>_m0scan.json | Metadata for M0 scan |

For the 6 subjects that were scanned with the ASL plus M0 protocol the tag will be acq-plusM0 and there is no separate M0 scan as the M0 was collected as first volume of the ASL.

### Anatomical scans

T1w and FLAIR files are located within the BIDS hierarchy in the anat folder. The defaced files are available for processing and these are named _rec-defaced_T1w and _rec-defaced_FLAIR

### Field Maps

Two echoes of the QSM MEGRE acquisition (available in the anat folder) have been copied over directly to the field map directory fmap for use in susceptibility distortion correction of the ASL scans. These scans are named _phase1 , _phase2, _magnitude1 and _magnitude2 and fall under case 2 of the BIDS specification (Two phase maps and two magnitude maps)

## Additional ASL fields

Additional information was appended to the BIDS metadata for the ASL acquisitions to enable them to conform to the BIDS standards. The values for these fields were determined from the MRI  Protocols directly. The  BIDS specifications for ASL can be found here https://bids-specification.readthedocs.io/en/stable/appendices/arterial-spin-labeling.html)

### PASL

ASL additional fields:

"EffectiveEchoSpacing": 0.0005,

"PostLabelingDelay": 2.75,

"LabelingDuration": 0.8,

"BolusCutOffFlag": "true",

"BolusCutOffTechnique": "QUIPSSII",

"BolusCutOffDelayTechnique": "QUIPSSII",

"BolusCutOffDelayTime": 0.8,

"RepetitionTimePreparation": 4,

"M0Type": "Separate"

M0 additional fields:

"IntendedFor": "ses-01/perf/sub-HML0077_ses-01_acq-prod_asl.nii.gz",

"RepetitionTimePreparation": 4

### PCASL

ASL additional fields:

"EffectiveEchoSpacing": 0.0002519,

"PhaseEncodingDirection": "j",

"ArterialSpinLabelingType": "PCASL",

"PostLabelingDelay": 1.95,

"LabelingDuration": 1.8,

"RepetitionTimePreparation": 4.2,

"M0Type": "Separate"

M0 additional fields:

"PhaseEncodingDirection": "j",

"IntendedFor": "ses-01/perf/sub-HML0001_ses-01_acq-pcasl_asl.nii.gz",

"RepetitionTimePreparation": 10

### Effective Echo Spacing

The effective echo spacing is required for optimal field map application. This is provided for the Siemens PASL acquisitions in the protocol pdf.

The effective echo spacing for the PCASL acquisition was obtained using a formula provided at the OSF site of the Spinoza Center at https://osf.io/hks7x which calculates this for Philips sites:

wfs = 3.5;

epifactor= 15;

acceleration = 2;

EESms = (((1000*wfs)/(434.215*(epifactor + 1))/acceleration))

EESms = 0.2519

# PAN-ASL-003: HPC Analysis

## BASIL Processing

BASIL v4.029-2-gc633891 which is bundled with FSL 6.0.7.4  is used for processing on the University of Arizona’s High Performance Computer. For both PCASL and PASL the following choices have been made.

- Use of a simple model that is recommended in the ASL White Paper (Alsop et al 2015) which assumes complete bolus arrival in the voxel. The main implications of this are that we use an Arterial Transit Time (ATT) default of 0s and T1 of 1.65 seconds for both tissue and blood. Also we use voxel calibration. This is specified by using the parameter --wp. BASIL will thus not attempt to estimate the ATT and the T1 as these are already fixed but it will still use a smooth spatial prior to perform spatial regularization of the CBF estimates (see https://fsl.fmrib.ox.ac.uk/fsl/fslwiki/BASIL/UserGuide#Spatial_regularization)

- There are arguments for and against using the Voxel-based calibration method with some researchers opting for use of a single M0 value derived from a region of interest like the CSF or WM. On balance however Pinto et al (2020) https://pubmed.ncbi.nlm.nih.gov/31605558/ recommend voxel calibration because it is simple to implement (thus less prone to error) and also intrinsically corrects for the coil sensitivity profile and so we have opted to use this approach.

- Use of FSL’s automated  fslanat pipeline for coregistration of the ASL data to MNI space and to perform tissue segmentation of the T1 image which is used by BASIL for partial volume correction.

- Perform susceptibility distortion correction using the first 2 echos of the Mult-echo GRE acquisition that was collected for QSM. We have used a customized version of  FSL’s fsl_prepare_fieldmap script to create a valid fieldmap which can be used in BASIL.

- In order to use the field map we need the echo spacing for the ASL acquisition. For PASL we are simply using what is provided by the Siemens Card i.e. 0.0005 seconds.  Phase Encoding Direction of PASL determined as "-y".

- For PCASL on JHU’s Philips we are using the OSF/Spinoza Center formula https://osf.io/hks7x to calculate echo spacing as 0.0002519. Phase Encoding Direction of PCASL determined as "+y"

- (f)We also perform motion correction using FSL’s MCFLIRT. This is specified with the –mc parameter

- (g) we also pass --pvcorr for  additional partial volume corrected  outputs.

### PASL

In addition to the above common steps, PASL processing in BASIL has some special requirements. These are listed below:

- For PASL, we are using QUIPSS II for fixing the label duration and so will pass the --fixbolus parameter to BASIL to reflect this; The Control comes before Label in sequence and so we use --iaf=ct

- For PASL, label duration(--bolus)  is 0.8s, TI (--tis)  is  2.75 s . Confusingly the TIS is actual stored in the BIDS Json file as "PostLabelingDelay" which is the BIDS specified way of doing this.

- TR (--tr) of the M0 is 4 seconds

A typical PASL command line call looks as follows:

oxford_asl --mc \
           --fixbat \
           --wp \

-o .../basiloutput

-i .../perf/sub-HML0182_ses-SESSION001_acq-prod_asl.nii.gz \
           --ibf=rpt \
           --rpts=10 \
           --fixbolus \
           --bolus=0.8 \
           --tis=2.75 \
           --fmap=.../desc-preproc_desc-rads_fieldmap.nii.gz
           --fmapmag=.../desc-magnitude_fieldmap.nii.gz \
           --fmapmagbrain.../desc-magnitude_desc-brain_fieldmap.nii.gz \
           --echospacing=0.0005 \
           --pedir=-y \
           --fslanat=.../fslanat_node/HML0182_struct.anat
           --iaf=ct
           --cmethod=voxel
           -c .../sub-HML0182_ses-SESSION001_acq-prod_m0scan.nii.gz \
           --tr=4 \

--pvcorr

### PCASL

In addition to the above common steps, PCASL processing in BASIL has some special requirements. These are listed below:

- For PCASL we pass the parameter --casl.

- For PCASL, label duration(--bolus)  is 1.8s. The  --tis parameter is calculated as the label_duration + the post labelling delay .  The "PostLabelingDelay" is stored in the the BIDS json and is 1.95 seconds  . The --tis parameter  is thus   3.75 s

- TR (--tr) of the M0 is 10 seconds

A typical PASL command line call looks as follows:

oxford_asl --mc \
           --fixbat \
           --wp \
           -o .../basiloutput \
           -i .../sub-HML0227_ses-SESSION001_acq-pcasl_asl.nii.gz \
           --ibf=rpt \
           --rpts=10 \
           --bolus=1.8 \
           --casl \
           --tis=3.75 \
           --fmap=.../desc-preproc_desc-rads_fieldmap.nii.gz \
           --fmapmag=.../fmapid-auto00000_desc-magnitude_fieldmap.nii.gz \
           --fmapmagbrain=..._desc-magnitude_desc-brain_fieldmap.nii.gz \
           --echospacing=0.0002519 \
           --pedir=y \
           --fslanat=.../fslanat_node/HML0227_struct.anat \
           --iaf=tc \ 
           --cmethod=voxel \
           -c .../sub-HML0227_ses-SESSION001_acq-pcasl_m0scan.nii.gz \
           --tr=10 \
           --pvcorr

# PAN-ASL-004: ASL Derivatives

After processing the ASL data with BASIL and ASLPREP, we obtain some derivative measures which are shared in the coree_chen_asl_measures table.

## BASIL CBF Derived Summary Measures

Using the perfusion maps created by BASIL we have derived CBF values by combining the Chemical shift mask created using MNE Python, the Gray matter and White matter masks calculated using FSL’s FAST algorithm (as part of the fsl_anat anatomical processing pipeline  https://web.mit.edu/fsl_v5.0.10/fsl/doc/wiki/fsl_anat.html) and the cortical masks derived by Freesurfer’s surface pipeline using mris_volmask (https://surfer.nmr.mgh.harvard.edu/fswiki/mris%25volmask )  to obtain tissue summaries of CBF in cortical GM and cerebral WM. These measures are provided in the subject’s native space using a combined transform from freesurfer’s conformed space to the native anatomical space (calculated using freesurfers tkregister2 command) and from native anatomical space to the perfusion space (calculated by BASIL’s pipeline).

| Measure | Description |
| --- | --- |
| basil_pvcorr_gm_chemshift.gmcort.perfusion_calib.Cerebral_Cortex | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cortical GM |
| basil_pvcorr_gm_chemshift.gmhemi.perfusion_calib.Left_Cerebral_Cortex | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cortical GM - Left hemisphere |
| basil_pvcorr_gm_chemshift.gmhemi.perfusion_calib.Right_Cerebral_Cortex | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cortical GM - Right hemisphere |
| basil_pvcorr_wm_chemshift.wmintra.perfusion_wm_calib.Cerebral_White_Matter | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cerebal WM |
| basil_pvcorr_wm_chemshift.wmhemi.perfusion_wm_calib.Left_Cerebral_White_Matter | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cerebral WM - Left hemisphere |
| basil_pvcorr_wm_chemshift.wmhemi.perfusion_wm_calib.Right_Cerebral_White_Matter | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask in Cerebral WM - Right hemisphere |

## BASIL CBF Measures in Arterial Territories

A 6-node arterial atlas in MNI152NLin6Asym space was created based  on the 32-node arterial atlas compatible with FSL (Atlas_182 at https://github.com/Chin-Fu-Liu/Arterial_Atlas/tree/main ) to determine summary CBF measures within major arterial regions in the cortical areas of the brain.

| 6 node cortical Region Name | Regions represented in 32 node atlas |
| --- | --- |
| LACA_CORT | ACAL, MLSL |
| RACA_CORT | ACAR, MLSR |
| LMCA_CORT | MCAFL, MCAPL,MCATL,MCAOL,MCAIL |
| RMCA_CORT | MCAFR, MCAPR,MCATR,MCAOR,MCAIR |
| LPCA_CORT | PCATL,PCAOL |
| RPCA_CORT | PCATR,PCAOR |

Refer to Arterial_Atlas/data/ArterialAtlasLables.txt at main · Chin-Fu-Liu/Arterial_Atlas · GitHub for labels.

This atlas was used in conjunction with the Chemical shift mask created using MNE Python and the Gray matter and White matter masks calculated using FSL’s FAST algorithm (as part of the fsl_anat anatomical processing pipeline  https://web.mit.edu/fsl_v5.0.10/fsl/doc/wiki/fsl_anat.html) to obtain ROI measures. For each node/roi a summary value is obtained using the GM and WM mask of BASIL’s PVC values.

These measures are provided in the subject’s native space using a combined transform from MNI152NLin6Asym space to the native anatomical space (calculated calculated by the fsl_anat pipeline) and from native anatomical space to the perfusion space (calculated by BASIL’s pipeline).

Provided below is an example for 1 region ACAL. Similar results will be provided for all 6 regions.

| Measure | Description |
| --- | --- |
| basil_pvcorr_gm_chemshift.arterial_32nodes.perfusion_calib.LACA_CORT | BASIL - Derived mean PVC GM CBF using Chemical Shift Mask with Arterial Region |
| basil_pvcorr_wm_chemshift.arterial_32nodes.perfusion_calib.LACA_CORT | BASIL - Derived mean PVC WM CBF using Chemical Shift Mask with Arterial Region |

# PAN-ASL-005: Exceptions

Certain subjects/sessions could not be successfully processed using the pipeline above or were removed to maintain validity and accuracy. These are delineated below:

| BIDS Participant | BIDS Session | Reason |
| --- | --- | --- |
| sub-HML0001 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0003 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0012 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0021 | ses-01 | No scan acquired |
| sub-HML0028 | ses-01 | No scan acquired |
| sub-HML0037 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0046 | ses-01 | ASL Plus MO acquisition |
| sub-HML0048 | ses-01 | ASL Plus MO acquisition |
| sub-HML0049 | ses-01 | ASL Plus MO acquisition |
| sub-HML0050 | ses-01 | ASL Plus MO acquisition |
| sub-HML0051 | ses-01 | ASL Plus MO acquisition |
| sub-HML0053 | ses-01 | No scan acquired |
| sub-HML0074 | ses-01 | Scan terminated early. ASL not acquired |
| sub-HML0114 | ses-01 | No scan acquired |
| sub-HML0145 | ses-01 | No scan acquired |
| sub-HML0148 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0168 | ses-01 | No scan acquired |
| sub-HML0180 | ses-01 | No scan acquired |
| sub-HML0183 | ses-01 | No scan acquired |
| sub-HML0185 | ses-01 | No scan acquired |
| sub-HML0186 | ses-01 | No scan acquired |
| sub-HML0195 | ses-01 | ASL Plus MO acquisition |
| Sub-HML0202 | ses-01 | Problem with Calibration M0 image. |
| sub-HML0240 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0272 | ses-01 | No scan acquired |
| sub-HML0278 | ses-01 | No scan acquired |
| sub-HML0283 | ses-01 | No scan acquired |
| sub-HML0288 | ses-01 | No scan acquired |
| sub-HML0292 | ses-01 | No scan acquired |
| sub-HML0293 | ses-01 | No scan acquired |
| sub-HML0294 | ses-01 | No scan acquired |
| sub-HML0295 | ses-01 | No scan acquired |
| sub-HML0331 | ses-01 | No scan acquired |
| sub-HML0333 | ses-01 | No scan acquired |
| sub-HML0344 | ses-01 | No scan acquired |
| sub-HML0355 | ses-01 | No scan acquired |
| sub-HML0358 | ses-01 | No scan acquired |
| sub-HML0363 | ses-01 | No scan acquired |
| sub-HML0367 | ses-01 | No scan acquired |
| sub-HML0369 | ses-01 | No scan acquired |
| sub-HML0374 | ses-01 | No scan acquired |
| sub-HML0381 | ses-01 | No scan acquired |
| sub-HML0396 | ses-01 | No scan acquired |
| sub-HML0401 | ses-01 | ASL not acquired |
| sub-HML0405 | ses-01 | No scan acquired |
| sub-HML0407 | ses-01 | ASL not acquired |
| sub-HML0409 | ses-01 | No scan acquired |
| sub-HML0424 | ses-01 | No scan acquired |
| sub-HML0434 | ses-01 | No scan acquired |
| sub-HML0438 | ses-01 | No scan acquired |
| sub-HML0441 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0448 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0452 | ses-01 | No scan acquired |
| sub-HML0453 | ses-01 | No scan acquired |
| sub-HML0456 | ses-01 | No scan acquired |
| sub-HML0463 | ses-01 | ASL not acquired |
| sub-HML0465 | ses-01 | No scan acquired |
| sub-HML0491 | ses-01 | No scan acquired |
| sub-HML0505 | ses-01 | ASL not acquired |
| sub-HML0526 | ses-01 | No scan acquired |
| sub-HML0530 | ses-01 | No scan acquired |
| sub-HML0537 | ses-01 | No scan acquired |
| sub-HML0561 | ses-01 | ASL not acquired |
| sub-HML0568 | ses-01 | No scan acquired |
| sub-HML0585 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0617 | ses-01 | No scan acquired |
| sub-HML0652 | ses-01 | No scan acquired |
| sub-HML0653 | ses-01 | No scan acquired |
| sub-HML0661 | ses-01 | Scan terminated early. ASL not acquired |
| sub-HML0663 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0666 | ses-01 | No scan acquired |
| sub-HML0667 | ses-01 | No scan acquired |
| sub-HML0670 | ses-01 | No scan acquired |
| sub-HML0685 | ses-01 | ASL not acquired |
| sub-HML0692 | ses-01 | No scan acquired |
| sub-HML0701 | ses-01 | No scan acquired |
| sub-HML0702 | ses-01 | ASL not acquired |
| sub-HML0710 | ses-01 | No scan acquired |
| sub-HML0714 | ses-01 | No scan acquired |
| sub-HML0720 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0731 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0735 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0738 | ses-01 | No scan acquired |
| sub-HML0759 | ses-01 | MEGRE not collected – Field Map not available |
| sub-HML0772 | ses-01 | No scan acquired |
| sub-HML0775 | ses-01 | No scan acquired |
| sub-HML0788 | ses-01 | No scan acquired |
| sub-HML0792 | ses-01 | No scan acquired |
| Sub-HML0819 | ses-01 | Problem with Calibration M0 image. |
| sub-HML0862 | ses-01 | No scan acquired |

# PAN-ASL-006: References

Liu, CF., Hsu, J., Xu, X. et al. Digital 3D Brain MRI Arterial Territories Atlas. Sci Data 10, 74 (2023). https://doi.org/10.1038/s41597-022-01923-0 (https://www.nature.com/articles/s41597-022-01923-0#citeas)

Zhang, Y. and Brady, M. and Smith, S. Segmentation of brain MR images through a hidden Markov random field model and the expectation-maximization algorithm. IEEE Trans Med Imag, 20(1):45-57, 2001. (https://web.mit.edu/fsl_v5.0.10/fsl/doc/wiki/FAST.html )

Jenkinson, M., Beckmann, C. F., Behrens, T. E. J., Woolrich, M. W., & Smith, S. M. (2012). FSL. NeuroImage, 62(2), 782–790. https://doi.org/10.1016/j.neuroimage.2011.09.015

Chappell MA, Groves AR, Whitcher B, Woolrich MW. Variational Bayesian inference for a non-linear forward model. IEEE Transactions on Signal Processing 57(1):223-236, 2009.

A.R. Groves, M.A. Chappell, M.W. Woolrich, Combined Spatial and Non-Spatial Prior for Inference on MRI Time-Series , NeuroImage 45(3) 795-809, 2009.

Chappell MA, MacIntosh BJ, Donahue MJ,Jezzard P, Woolrich MW. Partial volume correction of multiple inversion time arterial spin labeling MRI data, Magn Reson Med, 65(4):1173-1183, 2011.

Adebimpe, A., Bertolero, M., Dolui, S., Cieslak, M., Murtha, K., Baller, E. B., ... & Satterthwaite, T. D. (2022). ASLPrep: a platform for processing of arterial spin labeled MRI and quantification of regional brain perfusion. Nature methods, 19(6), 683-686

Alexandre Gramfort, Martin Luessi, Eric Larson, Denis A. Engemann, Daniel Strohmeier, Christian Brodbeck, Roman Goj, Mainak Jas, Teon Brooks, Lauri Parkkonen, and Matti S. Hämäläinen. MEG and EEG data analysis with MNE-Python. Frontiers in Neuroscience, 7(267):1–13, 2013. doi:10.3389/fnins.2013.00267.

Fischl, B. (2012). FreeSurfer. Neuroimage, 62(2), 774-781.