Nucleic Acid Extraction Using the MagMAX™ Pathogen RNA/ DNA Kit on a Magnetic

Particle Processor Worksheet

This worksheet to be used in conjunction with Nucleic Acid Extraction using the MagMAX™ Pathogen RNA/DNA Kit on a Magnetic Particle Processor Protocol

Date: ____________________________Operator: ______________________________________________

Project/ Accession Number: ________________________________________________________________

Step 1.  MagMAX™ Pathogen RNA/DNA Kit Extraction Plate Layout

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |  |  |  |
| G |  |  |  |  |  |  |  |  |  |  |  |  |
| H |  |  |  |  |  |  |  |  |  |  |  |  |

Record sample ID numbers, sample information, control information (Positive Extraction Control (PEC), Negative Extraction Control (NEC)), etc. in the wells above.

Step 2.  Control Information

Note: For multiplex assays, the Extraction Control can serve as both the NEC and the PEC.

| Control | Lot # | Expiration Date |
| --- | --- | --- |
| Extraction Control(s) |  |  |
| Xeno™ Internal Positive RNA Control |  |  |
| Additional Control if needed |  |  |

Step 3.  Working Stock (WS) Buffer Preparation

Fill in Company, Lot #, Date prepared, and Initials.

Step 4.  Extraction Reagent Preparation

Fill in Lot # and Initials.

Note:  Enter the number of samples in the highlighted well under the “Rxns”.

Volumes needed will automatically be calculated with an additional 10% overage.

Step 5. Extraction Plate Preparation/ Procedure

See NVSL-SOP-0644 for detailed instructions including programming of the magnetic particle processor.

1.  Reagent Preparation									     Complete

| Prepare extraction reagents in a hood in the clean room according to Steps 3 and 4 above. Note: Xeno™ RNA Control should be added to the Lysis/Binding Solution in the extraction room. |  |
| --- | --- |

2.  Plate Preparation

| Microplate 1. Beads, Sample, Lysis, and Xeno™ RNA |  |  |
| --- | --- | --- |
| 1. | In the extraction room, add 20µl Bead Mix to Deep Well Microplate. |  |
| 2. | Add 200µl of each prepared sample (100µl for ASF whole blood samples) to Deep Well Microplate. Cover and place on plate shaker for ~1 minute at moderate speed. |  |
| 3. | Add 400µl of Lysis/Binding Solution WS to each sample. Place on plate shaker and shake for ~1 minute at moderate speed. |  |
| Microplates 2 and 3. Wash Solution 1 WS |  |  |
| 4. | Add 300µl of Wash Solution 1 WS to Deep Well Microplate. |  |
| Microplates 4 and 5. Wash Solution 2 WS |  |  |
| 5. | Add 450µl of Wash Solution 2 WS to Deep Well Microplate. |  |
| Microplate 6. Elution |  |  |
| 6. | Add 90µl of Elution Buffer to Standard (or Deep Well) Microplate. |  |

3.  Load Magnetic Particle Processor (Note: Plates are loaded in reverse order of procedure.)

| Position 7 | Load empty Standard (or Deep Well) Microplate and Deep Well Rod Cover |  |
| --- | --- | --- |
| Position 6 | Load Standard (or Deep Well) Microplate 6 (Elution) |  |
| Position 5 | Load Deep Well Microplate 5 (Wash Solution 2 WS) |  |
| Position 4 | Load Deep Well Microplate 4 (Wash Solution 2 WS) |  |
| Position 3 | Load Deep Well Microplate 3 (Wash Solution 1 WS) |  |
| Position 2 | Load Deep Well Microplate 2 (Wash Solution 1 WS) |  |
| Position 1 | Load Deep Well Microplate 1 (Beads, Sample, Lysis, and Xeno™ RNA) |  |

4.  Start Run

| Start run on the magnetic particle processor. |  |
| --- | --- |
| When run is completed, remove Elution Plate from magnetic particle processor. Eluted nucleic acid can be transferred to an alternate storage container if desired. Use immediately or store the nucleic acid at -70 (±10°C) until use. |  |
| Complete worksheet and enter in appropriate notebook/ data collection log. |  |

5.  Deviations (Any deviations must be pre-approved, noted, and initialed in the box below.)

Signature________________________________________________Date__________________________