Nucleic Acid Extraction Using the INDICAL BIOSCIENCES

IndiMag Pathogen Kit on a Magnetic Particle Processor Worksheet

Worksheet to be used in conjunction with Nucleic Acid Extraction Using the INDICAL BIOSCIENCE IndiMag Pathogen Kit on a Magnetic Particle Processor Protocol

Date: ____________________________Operator: ______________________________________________

Project/Accession Number: ________________________________________________________________

Step 1.  Working Stock Reagent (WS) Preparation

Fill in Company, Lot #, Date prepared, and Initials.

Step 2.  Extraction Reagent Preparation

Fill in Lot # and Initials.  Enter the number of samples in the highlighted well under the “Rxns”.

Enter the desired volume of Xeno™ RNA Internal Control in the highlighted well under the “Vol/rxn (µl)” column.  Volumes needed will automatically be calculated with an additional 10% overage.

Step 3. Extraction Plate Preparation/ Procedure

1.  Reagent Preparation									     Complete

| Prepare extraction reagents in a hood in the clean room according to Steps 1 and 2 above. |  |
| --- | --- |

2.  Plate Preparation

| Microplate 1. Lysate |  |  |
| --- | --- | --- |
| 1. | In the extraction room, add 20µl Proteinase K to Deep Well Microplate. |  |
| 2. | Add 200µl of each prepared sample (100µl for ASF whole blood samples) to Deep Well Microplate. |  |
| 3. | Vortex Buffer VXL Mixture (prepared in Step 2 above) thoroughly for 30 seconds. Add 500µl Buffer VXL Mixture to each well. Vortex Mixture every 3 minutes. |  |
| Microplate 2. Wash 1 |  |  |
| 4. | Add 700µl of Buffer AW1 WS to Deep Well Microplate. |  |
| Microplate 3. Wash 2 |  |  |
| 5. | Add 700µl of Buffer AW2 WS to Deep Well Microplate. |  |
| Microplate 4. Wash 3 |  |  |
| 6. | Add 750µl of 100% Ethanol to Deep Well Microplate. |  |
| Microplate 5. Elution |  |  |
| 7. | Add 100µl of Buffer AVE to Standard (or Deep Well) Microplate. |  |

3.  Load Magnetic Particle Processor (Note: Plates are loaded in reverse order of procedure.)

| Position 6 | Load empty Standard (or Deep Well) Microplate and Deep Well Rod Cover |  |
| --- | --- | --- |
| Position 5 | Load Standard (or Deep Well) Microplate 5 (Elution) |  |
| Position 4 | Load Deep Well Microplate 4 (Wash 3) |  |
| Position 3 | Load Deep Well Microplate 3 (Wash 2) |  |
| Position 2 | Load Deep Well Microplate 2 (Wash 1) |  |
| Position 1 | Load Deep Well Microplate 1 (Lysate) |  |

4.  Start Run

| Start run on the magnetic particle processor. Recommended protocol: BS96 cador v2 |  |
| --- | --- |
| When run is completed, remove elution plate from magnetic particle processor. Eluted nucleic acid can be transferred to an alternate storage container if desired. Use immediately or store the nucleic acid at -70 (±10°C) until use. |  |
| Complete worksheet and enter in appropriate notebook/data collection log. |  |

5.  Deviations (Any deviations must be pre-approved, noted, and initialed in the box below.)

Signature________________________________________________Date__________________________