African Swine Fever real-time PCR Assay on the Applied Biosystems™ QuantStudio™ 5 Real-Time PCR System Using the Quantabio UltraPlex® 1-Step ToughMix® Low ROX™ (4X) Worksheet

This worksheet to be used in conjunction with Preparation, Performance, and Interpretation of the African Swine Fever rPCR Assay on the Applied Biosystems® QuantStudio 5 Real-time PCR System Protocol

Date: _____________Operator: ________________ Project: ______________________________________

# Step 1.  Master mix preparation (clean room)

# ASF master mix (Use with 96-well extraction method and Xeno™ RNA)

# Note: Double click on the table, enter number of reactions in the highlighted well, and the volumes will be calculated.

The Xeno™ IEC probe is labeled with a LIZ fluorophore but is detected under the Cy5 channel on the ABI QS5.

Step 2.  Addition of the master mix, template, and controls

| Step |  | Check |
| --- | --- | --- |
| 1. | Add 20 µl of master mix to each well of a 96-well PCR plate in the clean room. |  |
| 2. | Add 5 µl of nuclease-free water to the no template control (NTC) well in the clean room. |  |
| 3. | In the extraction room, add 5 µl of template DNA to each of the sample wells. |  |
| 4. | Add 5 µl of the extraction control(s) to the appropriate control well(s). |  |
| 5. | Add 5 µl of positive amplification control (PAC) to the appropriate control well(s). |  |
| 6. | Briefly centrifuge plate. |  |

Step 3.  Running the Assay

| Platform | Standard Cycling Profile | Check or Initial |
| --- | --- | --- |
| Applied Biosystems QuantStudio 5 | - 1. 50ºC 10 minutes - 2. 95°C 3 minutes - 3. 95ºC 10 seconds - 4. 60ºC 30 seconds - 5. Repeat steps 3-4 a total of 45 cycles |  |

# Any variations from this procedure are noted, dated and initialed below or on back of this sheet.

Signature(s): ___________________________________            Date: ____________________